RFID BASED SECURITY AND AUTHENTICATION SYSTEM

A MINI PROJECT REPORT

- Atul Jalindar Gaikwad

- Amitkumar Shingankar

- Rohan Sunil Kamthe

Submitted by

TO THE SAVITRIBAI PHULE PUNE UNIVERSITY, PUNE

In partial fulfillment for the award of the Degree of

Bachelor of Engineering

In

ELECTRONICS ENGINEERING

Under the Guidance of

Prof. N. P. Mawale sir

DEPARTMENT OF ELECTRONICS AND TELECOMMUNICATION ENGINEERING

All India Shri Shivaji Memorial Society’s

College of Engineering, Pune-1

APRIL- 2018.

CERTIFICATE

This is to certify that the Mini Project Report entitled:

RFID BASED SECURITY AND AUTHENTICATION SYSTEM			                 Submitted by: Mr.  Atul J. Gaikwad 				Seat No. T150213818

Mr.  Amitkumar Shingankar 			Seat No. T150213867

Mr.  Rohan S. Kamthe				Seat No. T150213832

is a bonafide record of the work carried out by him/her towards the partial fulfillment of the requirements of Savitribai Phule Pune University, for the award of degree of Bachelor of Engineering in Electronics Engineering under my/our supervision and guidance.

Signature of the Guide

Name of the Guide: N. P. Mawale

Designation:

| Dr. D. S. Bormane | Dr. D. S. Bormane |
| --- | --- |
| HOD | Principal |
| Electronics & TC Engineering | AISSMS COE, Pune-41 |

Date:

Place:

________________________________________________________________________

This report has been examined by us as per the SAVITRIBAI PHULE PUNE UNIVERSITY, PUNE, requirements at AISSMS COE Pune-1 on Date: _______________

| Signature | Signature |
| --- | --- |
| Name: | Name: |
| Internal Examiner | External Examiner |

Acknowledgement

The satisfaction that accompanies the successful completion of this project would be incomplete without the mention of the people who made it possible, without whose constant guidance and encouragement would have made efforts go in vain. We consider myself privileged to express gratitude and respect towards all those who guided us through the completion of this project.

We convey thanks to my project guide Prof. N. P. Mawale for providing encouragement, constant support and guidance which was of a great help to complete this project successfully. He helped us in, how to do things easily and how to manage it which helps more while actual working on project. We have improved our presentation skills by their comments and tips.

We would also like to thank the role of the staff of Electronics Laboratory, they helped us in necessary material to complete the simulation and understanding the concepts more elaborately.

Last but not the least, we wish to thank our parents for financing our studies in this college as well as for constantly encouraging us to learn engineering. Their personal sacrifice in providing this opportunity to learn engineering is gratefully acknowledged.

Thank you.

DECLARATION

We hereby declare that this submission is our work and that, to the best of our knowledge and belief, it contains no material previously published or written by another person nor material which has been accepted for award of any other degree or diploma of the university or other institute of higher learning, except where due acknowledgement has been made in the text.

Place:

Date:

Signature of Student

Name: Atul Jalindar Gaikwad

PRN: 71532879E

Signature of Student

Name: Amitkumar Shingankar

PRN: 71533287C

Signature of Student

Name: Rohan Sunil Kamthe

PRN: 71532973B

CONTENTS

Table of Contents……………………..…..........................................................................................i

Abstract............................................................................................................................... ii

List of Figures.....................................................................................................................iii

List of Tables……............................................................................................................. iv

List of Acronyms.................................................................................................................v

Chapter No			Title of Chapter			             Page no.

1                   Introduction

  - Introduction...........................................................................1

  - Motivation.............................................................................2

  - Problem Statement................................................................2

2                    Block Diagram and Description

  - Block diagram...................................................................... 5

    - Block Diagram Description................................... 5

  - Hardware and  Feature......................................................... 6

    - PIC18F4520............................................................. 6

    - LCD…………………..............................................7

    - L293D...................................................................... 8

    - DC MOTOR............................................................ 9

    - Power Supply.......................................................... 9

    - LED……………....................................................10

    - Resistor .................................................................10

    - Capacitor................................................................11

  - Software and Feature...........................................................12

3                     Circuit Diagram and Description

  - Circuit diagram...................................................................14

4.                  Literature Survey and Selection of Components

  - RFID operation and working……………………..………………..15

  - Transformer Rating………………………………………………

  - Regulator 7805 & 7812

  - 1N4007 rectifier diode

  - L293D motor driver working…………………………………………

  - DC Motor Working…………………………………………………...

  - LCD display……………………………….

  - POT for contrast of lcd display……………..

  - Component Pricing .........................................................................19

5                       Analysis and simulation

5.1   Simulation Result..............................................................21

5.2   Software Design Aspects..................................................21

5.2.1 Algorithm ...............................................................21

5.2.2 Flow Chart.............................................................22

5.2.3 Program..................................................................23

5.3 PCB Layout........................................................................27

5.4   Art work............................................................................27

5.4.1 Materials..................................................................27

5.4.2 Patterning.................................................................28

5.4.3 Etching.....................................................................28

5.4.4 Lamination...............................................................29

5.4.5 Drilling....................................................................29

5.4.6 Damage....................................................................30

5.4.7 Exposed Conductor Plating and Coating.................30

5.4.8 Solder Resist............................................................30

5.4.9 Component Mounting..............................................31

5.4.10 Soldering...............................................................32

5.4.11 Precautions............................................................33

6                        Conclusion and Future Scope

  - Results.................................................................................35

  - Advantages..........................................................................35

  - Limitations..........................................................................36

  - Conclusion...........................................................................36

  - Application..........................................................................36

  - Future Scope........................................................................36

- Reference………………………………………………

ABSTRACT

Now a days all the data of companies, colleges or any organizations is stored in digital format. So the safety of this precious data is the major concern of any authorities.

In this project, the low cost RFID Based Security and Authentication System has been designed with using PIC 18F4520. It checks the authorized or unauthorized person and displayed on LCD.

The concern is for the physical property and also for the intellectual property. So it is important to secure it from unauthorized or unwanted person.

For this reason, by installing this system in organization, only the authorized person with a valid RFID tag is allowed into the secured premises.

In such a way, unauthorized persons can be caught which will surely improve the security level in the organization.

List of figures

| Figure no. | Name of figure | Page No. |
| --- | --- | --- |
| Figure no.2.1.1 | Block diagram of main design | 5 |
| Figure no.2.2 | Pin diagram of PIC18F4520 | 7 |
| Figure no.2.2.2 | Pin diagram of L293D | 8 |
| Figure no.2.2.3 | 16x2 LCD | 9 |
| Figure no.2.2.6 | Capacitor | 10 |
| Figure no.2.2.7 | Resistor | 10 |
| Figure no.2.2.8 | RFID Reader | 11 |
| Figure no.3.1(a) |  | 14 |
| Figure no.3.2(b) |  | 15 |
| Figure no.4.1 |  | 17 |
| Figure no.4.4.1 |  | 18 |
| Figure no.4.1.2 |  | 18 |
| Figure no.5.1 | Simulation result | 19 |
| Figure no.5.3 | PCB layout | 27 |
| Figure no.5.4.3 | Etching | 29 |
| Figure no.5.4.9 | Components mounting | 32 |
| Figure no.5.4.10 | Soldering | 33 |
| Figure no.6.1 | Result | 35 |

Why we use 1N4007 Diodes?

1N400X diodes are used as rectifiers for low frequency having big capacitance at the junction, other diodes have less capacitance value therefore they have quick ON –OFF time.

These 1N4007 diodes are usually slow. To measure the voltage drop across the diode connect a load resistance across the cathode terminal and other terminal of the load is connected to negative terminal of the battery. Then the drop across the diode is 0.7 volts.

Regulator:

The purpose of regulator is to maintain constant voltage. For positive voltage output use LM78XX, XX indicates value of output voltage and 78 indicates positive output. For negative voltage output use LM79XX, 79 indicate negative voltage and XX indicates value of output. To get positive 5V regulated output use LM7805.To get negative 5V regulated output use LM7905. To get positive 12V regulated output use LM7812

Filtering:

The output of the rectifier is pulsating DC. This pulsating DC is converted into pure DC using filter. The filter used in this circuit is LC filter, but practically we won’t use inductor because it is bulky. The value of capacitor depends upon the output voltage and output current. To calculate the value of capacitor the following formulae is used

Q = C x V equation (1)

Q = C x IR

Q = I x RC

Q = I x T equation (2)

Substitute equation (2) in equation (1)

I x T = C x V

C = (I x T) / V

Here output voltage is V = 5 DC and V= 12V DC

Here output current is I = 1 amps

EM-18 RFID Reader:

This module directly connects to any microcontroller UART or through a DB9 converter to PC. It gives UART/Wiegand26 output. This RFID Reader Module works with any 125 KHz RFIDtags 

Specifications

- 5VDC through USB (External 5V supply will boost range of the module)

- Current: <50mA

- Operating Frequency: 125Khz

- Read Distance: 10cm

- Size of RFID reader module: 32mm(length) * 32mm(width) * 8mm(height)

L293D Motor Drive:

The L293D is popular motor driver IC that is usable from 6 to 12 V at up to 1A total output current makes it such more convenient

Features

The L293D is a popular motor driver IC that is usable from 6 to12V, at up to 1A total output current. By itself, the IC is somewhat difficult to wire and use, but the Compact L293D Motor Driver makes it much more convenient to use.

Board Special Features

- Four motor direction indicator LEDS

- Schottky EMF-protection diodes

- Socket pin connectors for easy logic interfacing

- Enable pins are user accessible.

L298 Features

1n4007